# Performance Smoke Test Report (Teacher Dashboard)

Date: November 30, 2025

System under Test: Teacher Performance Dashboard

Test Type: Smoke Test (Sanity Check)

Peak Load: 3 Concurrent Users (Minimal)

Prepared By: MD SAMI ALAM

## 1. Executive Summary

Overall Status: ✅ Pass / Healthy

The Smoke Test applied a minimal load of 3 Virtual Users (VUs) to verify that the Teacher Dashboard is accessible and functioning correctly without load.

Conclusion: The system is healthy. All 354 requests were processed successfully with zero errors. However, similar to other tests, the functional check pass rate was 66.7%, indicating potential data gaps in the test environment.

### Key Statistics

- Peak VUs: 3

- Total Requests: 354

- Average Throughput: ~5.8 RPS

- Response Time (p95): 24ms (Excellent baseline)

- HTTP Failure Rate: 0.0% (Perfect availability)

- Functional Check Rate: 66.7% (Consistent data issue)

## 2. Aggregate Metrics Summary

The following table summarizes the system's performance during this baseline check.

| Metric | Value | Status |
| --- | --- | --- |
| VUs | 3 | ✅ Target Met |
| Avg Throughput | 5.82 RPS | ✅ Normal for load |
| Response Time (avg) | 14ms | ✅ Excellent |
| Response Time (p95) | 24ms | ✅ Excellent |
| Max Response Time | 71ms | ✅ Excellent |
| Error Rate (HTTP) | 0.0% | ✅ Perfect |
| Check Pass Rate | 66.7% | Minor Issue |

## 3. Detailed Findings

### ✅ System Availability

- 100% Success: There were no HTTP 4xx or 5xx errors. The web server and application logic are correctly deployed and accessible.

### ✅ Performance Baseline

- Fast Response: The 95th percentile response time was 24ms, which is virtually instant. This confirms that the application code itself is performant when not under load. The severe latencies seen in load tests are purely a result of concurrency/resource contention, not inefficient single-thread code execution.

### ⚠️ Data Validation

- Check Failures: The 66.7% pass rate mirrors the Student Dashboard results. This strongly suggests a systemic issue with the test data seeding (e.g., missing records for specific test scenarios) rather than a bug in the dashboard code itself.

## 4. Recommendations

- Proceed to Load Testing: The system is fundamentally sound. Proceed with higher load tiers.

- Investigate Check Failures: Review the test scripts and database seed data to resolve the 33% check failure rate, ensuring that future tests accurately reflect functional health.

# Performance Load Test Report

# (Teacher Dashboard)

System under Test: Teacher Performance Dashboard

Test Type: Load Test (Step Stress)

Date: November 29, 2025

Load Tiers: 1,000 / 2,000 / 4,000 VUs

Overall Status: ⚠️ Operational (Limit Reached at ~1,200 Users)

## 1. Executive Summary

This test evaluated the Teacher Dashboard under escalating load. The results are strikingly similar to the Student Dashboard tests, suggesting a shared architectural bottleneck. The system handled 1,000 VUs with acceptable performance but began to show signs of stress at 1,200 VUs and failed critically at 2,000+ VUs.

Conclusion: The Teacher Dashboard is production-ready for loads up to 1,000 concurrent users. Beyond 1,200 users, response times degrade significantly, and error rates increase, rendering the system unstable.

### Key Statistics

- Certified Capacity: 1,000 Concurrent Users (Stable).

- Breaking Point: ~1,200 Users.

- Critical Failure: > 2,000 Users (High latency & errors).

## 2. Scalability Matrix (Comparative Analysis)

The following table compares system behavior across the three distinct load tiers.

| Metric | Tier 1: 1,000 VUs | Tier 2: 2,000 VUs | Tier 3: 4,000 VUs |
| --- | --- | --- | --- |
| Status | 🟡 PASS (Warnings) | 🔴 FAIL | 🔴 CRITICAL FAIL |
| Throughput (RPS) | 525 RPS (Peak) | 410 RPS (Saturation) | 280 RPS (Collapse) |
| Response Time (p95) | 2.0s (Good) | 6.0s (Slow) | 27.0s (Unusable) |
| Error Rate (HTTP) | 5.2% | ~10% | > 15% |
| Connection Wait | ~0.6s | ~3.0s | ~15.0s |

## 3. Detailed Findings by Tier

### ✅ Tier 1: 1,000 Concurrent Users (The "Safe Zone")

- Observation: At 1,000 users, the system maintained a throughput of 525 RPS with a 95th percentile response time of roughly 2.0 seconds. This is within acceptable limits for a data-heavy dashboard.

- Issues: A 5.2% error rate was observed even at this tier. This is higher than the Student Dashboard's baseline, suggesting the Teacher Dashboard queries might be more complex or resource-intensive.

### ❌ Tier 2: 2,000 Concurrent Users (The "Failure Zone")

- Observation: As load increased to 2,000 users, the system could not scale. Throughput dropped to 410 RPS, indicating resource contention.

- Bottleneck: Connection times (http_req_connecting) and blocked times (http_req_blocked) increased significantly, pointing to a backlog in the web server's ability to accept new connections.

### ⛔ Tier 3: 4,000 Concurrent Users (The "Collapse Zone")

- Observation: The system was effectively unresponsive. Response times hit 27s - 48s, and a large percentage of requests failed.

- Verdict: The system cannot support 4,000 users in its current configuration.

## 4. Root Cause Analysis

1. Shared Architectural Limits

- The breaking point (~1,200 VUs) and failure mode (connection backlog) are nearly identical to the Student Dashboard tests. This confirms that the bottleneck is likely at the infrastructure level (Load Balancer, Web Server configuration, or Database connection pool) rather than specific application code.

2. Heavier Queries?

- The higher baseline error rate (5.2% vs ~0.1-0.2% for Student) suggests that Teacher Dashboard endpoints might be performing heavier aggregations (e.g., calculating class averages vs single student grades), putting more strain on the database per request.

## 5. Recommendations

- Investigate Query Performance: Analyze the database queries used by the Teacher Dashboard. Are they properly indexed? Are complex aggregations being performed in real-time that should be cached?

- Infrastructure Tuning: As with the Student Dashboard, increasing worker_connections and file descriptor limits is a critical first step.

- Capacity Planning: Treat 1,000 users as the hard limit for now. Implement rate limiting to protect the system from surges beyond this point.

# Performance Stress Test Report (Teacher Dashboard)

System under Test: Teacher Performance Dashboard

Test Type: Stress Test (Ramp-up to Failure)

Date: November 30, 2025

Peak Load: 5,000 Concurrent Users

Overall Status: 🔴 Critical Failure (System Overwhelmed)

## 1. Executive Summary

The Teacher Dashboard was subjected to a stress test ramping up to 5,000 concurrent users. The system failed to support this load, exhibiting severe latency, connection timeouts, and a high volume of functional errors.

Conclusion: The system is currently not capable of supporting 5,000 concurrent teacher sessions. Performance remains stable only up to approximately 1,000 - 1,200 VUs. Beyond this, the application server becomes saturated, leading to a cascading failure where requests queue up until they time out.

### Key Statistics

- Peak VUs: 5,000

- Average Throughput: 413 RPS (Significantly throttled)

- Response Time (p95): 8.0s (High latency)

- Max Response Time: 60s (Timeout limit reached)

- HTTP Failure Rate: 6.0%

- Functional Check Rate: 62.0% (Data integrity compromised)

## 2. Aggregate Metrics Summary

The following table summarizes the system's performance over the entire duration of the stress test.

| Metric | Value | Status |
| --- | --- | --- |
| Max VUs | 5,000 | 🔴 Stress Limit |
| Avg Throughput | 413.13 RPS | 🔴 Low / Saturated |
| Response Time (avg) | 2.0s | Degraded |
| Response Time (p95) | 8.0s | 🔴 Unacceptable |
| Max Response Time | 1m (60s) | 🔴 Critical Timeout |
| Error Rate (HTTP) | 6.0% | 🔴 Failure |
| Check Pass Rate | 62.0% | 🔴 Critical Failure |

## 3. Detailed Findings

### ❌ Latency & Timeouts

- Extreme Delays: While the average response time was ~2 seconds, the tail latency was severe. The 95th percentile was 8 seconds, and the 99th percentile reached 22 seconds.

- Request Queuing: The maximum http_req_blocked time was 58 seconds. This indicates that requests were sitting in a queue within the OS or Load Balancer for nearly a minute before the application could even attempt to process them.

### ❌ Functional Reliability

- Data Integrity Loss: Only 62% of functional checks passed. This is a critical issue for a Teacher Dashboard, where accuracy is paramount. Teachers would likely see partial page loads, missing student data, or generic error pages.

### ⚠️ Throughput Bottleneck

- Low Processing Capacity: Despite the high load (5,000 users), the server could only process 413 requests per second. This is lower than the Student Dashboard's stress test throughput (~520 RPS), reinforcing the theory that Teacher Dashboard queries are more resource-intensive per request.

## 4. Root Cause Analysis

1. Connection Queue Overflow

- Evidence: The massive blocked (58s) and connecting (23s) times confirm that the web server's backlog queue was full. New connections were being rejected or held until they timed out.

2. Expensive Database Queries

- Evidence: The lower throughput compared to the Student Dashboard (413 vs 520 RPS) suggests that the database operations for teachers (likely aggregations of class data) are locking rows or consuming excessive CPU, slowing down the entire request pipeline.

## 5. Recommendations

- Query Optimization: Perform a deep dive into the SQL queries generated by the Teacher Dashboard. Look for full table scans or missing indexes on class/grade tables.

- Aggressive Caching: Teacher views (like class averages) do not change every second. Implement short-term caching (e.g., 1-5 minutes) for these expensive aggregations to reduce database load.

- Rate Limiting: Protect the system by limiting the number of active requests per teacher or per IP. It is better to slow down a few heavy users than to crash the dashboard for everyone.

# Performance Spike Test Report

# (Teacher Dashboard)

Date: November 30, 2025

Test Type: Spike Test (Sudden Surge)

Peak Load: 4,000 Concurrent Users (Instant Ramp-up)

Overall Status: 🔴 Critical Failure / System Collapse

## 1. Executive Summary

The system was subjected to a sudden spike in traffic, ramping up to 4,000 Virtual Users (VUs) in a very short window. Similar to the Student Dashboard, the Teacher Dashboard failed to handle this surge, leading to immediate service unavailability.

Conclusion: The system lacks the elasticity to handle sudden spikes. The rapid influx of users caused the connection queue to fill up immediately, preventing new users from accessing the dashboard.

### Key Statistics

- Peak Load: 4,000 VUs

- HTTP Failure Rate: 0.9% (Lower than expected, but misleading due to timeouts)

- Functional Check Rate: 63.5% (Significant failure in user flows)

- Connection Time (avg): 0.6s (High network latency/backlog)

- Blocked Time (avg): 0.8s (Thread pool exhaustion)

## 2. Aggregate Metrics Summary

The following table summarizes the system's performance during the spike event.

| Metric | Value | Status |
| --- | --- | --- |
| Peak VUs | 4,000 | 🎯 Target Reached |
| Avg Throughput | ~525 RPS | 🔴 Low (Throttled) |
| Response Time (p95) | 3.6s | High |
| Max Response Time | 1m (53s) | 🔴 Timeout |
| Blocked Time (Max) | 53s | 🔴 Critical |
| Error Rate (HTTP) | 0.9% | 🔴 Critical Failure |
| Check Pass Rate | 63.5% | 🔴 Critical Failure |

## 3. Detailed Findings

### ❌ Connection Storm

- Blocked & Connecting: The average time spent just waiting to send a request (http_req_blocked + http_req_connecting) was high, with some requests blocked for up to 53 seconds. This confirms that the server's "accept queue" was full.

- TLS Handshake Lag: The average TLS handshake took 89ms, which is reasonable, but the maximum was 53s, indicating severe CPU saturation during the initial spike.

### ❌ Functional Failure

- Low Check Rate: Only 63.5% of functional checks passed. This means that even if a request technically succeeded (HTTP 200), the dashboard often failed to load critical data components.

### ⚠️ Throughput Limit

- RPS Cap: The system plateaued at roughly 525 RPS, matching the limit seen in other tests. This confirms a hard ceiling on the number of requests the current infrastructure can handle per second.

## 4. Root Cause Analysis

1. Inadequate Auto-Scaling

- The system could not scale up fast enough to meet the demand of 4,000 users arriving almost simultaneously.

2. Synchronous Blocking

- The high blocked times suggest that the web server threads were exhausted immediately, leaving new connections in a wait state.

## 5. Recommendations

- Queueing/Throttling: Implement a "Waiting Room" or strict rate-limiting at the Load Balancer level to prevent the server from becoming overwhelmed during a spike.

- Pre-Warming: For known high-traffic events (e.g., report card release), pre-scale the infrastructure rather than relying on reactive auto-scaling.

- Asynchronous Processing: Offload heavy dashboard calculations to background jobs where possible, so the main web request can return quickly.

# Performance Soak Test Report

# (Teacher Dashboard)

Test Type: Soak Test (Endurance)

Date: December 1, 2025

Load: 500 Concurrent Users (Sustained)

Overall Status: ⚠️ Mechanically Stable / Functionally Degraded

## 1. Executive Summary

The endurance test confirms that the infrastructure is stable over time but suffers from significant functional degradation. The server maintained steady throughput and low latency for the vast majority of the test, but a substantial portion of requests failed business logic validation.

Conclusion: The server infrastructure (CPU/RAM/Network) is robust enough to handle sustained loads of 500 concurrent teachers. However, the application logic or data layer is failing to return valid content for 33.3% of requests, indicating a severe reliability issue that must be fixed before deployment.

### Key Statistics

- Total Requests: ~4.8 Million

- Average Throughput: 642 RPS

- Response Time (p95): 403ms (Excellent latency)

- HTTP Failure Rate: 0.1% (Infrastructure is healthy)

- Functional Check Rate: 66.7% (Application logic is failing)

## 2. Aggregate Metrics Summary

The following table summarizes the system's performance over the entire duration of the soak test.

| Metric | Value | Status |
| --- | --- | --- |
| VUs (Max) | 500 | ✅ Target Met |
| Total Data | 4.46 GB | - |
| Avg Throughput | 641.77 RPS | ✅ High & Stable |
| Response Time (avg) | 267ms | ✅ Fast |
| Response Time (p95) | 403ms | ✅ Fast |
| Error Rate (HTTP) | 0.1% | ✅ Excellent |
| Check Pass Rate | 66.6% | 🔴 Major Issue |

## 3. Detailed Findings

### ✅ Infrastructure Stability

- Consistent Performance: The 95th percentile response time held steady at 403ms, and the average was 267ms. There were no signs of memory leaks or creeping latency over the duration of the test.

- High Availability: With an HTTP error rate of only 0.1%, the web servers and load balancers proved they can handle 500 concurrent connections without dropping packets or crashing.

### ❌ "Soft" Failures (Data Integrity)

- 33% Failure Rate: The Check Pass Rate (66.6%) is identical to the Student Dashboard's soak test result. This strongly points to a shared backend issue where valid HTTP 200 OK responses are returned with:

  - Missing or empty JSON payloads.

  - Generic "Data not available" messages.

  - Incomplete HTML rendering.

## 4. Root Cause Analysis

1. Shared Backend Bottleneck

- The fact that both Student and Teacher dashboards exhibit the exact same 66.6% success rate suggests a common dependency—likely a shared microservice or database view—that is failing silently under sustained load.

2. Data Consistency

- It is possible that the test data set is incomplete, leading to valid requests for specific teacher/student IDs returning empty results, which the test script flags as a check failure.

## 5. Recommendations

- Debug "Soft" Failures: Inspect application logs specifically for the requests that returned 200 OK but failed content validation. Determine if this is an application error or a test data issue.

- Verify Test Data: Ensure the database is fully seeded with complete records for all 500 virtual users to rule out data gaps as the cause of the 33% failure rate.

- Deploy with Caution: While the infrastructure is ready, the application logic is not. Do not deploy until the functional check pass rate is > 99%.

# Performance Breakpoint Test Report (Teacher Dashboard)

Test Type: Breakpoint Test (Ramp-up to Failure)

Date: November 30, 2025

Peak Load: 1,500 Concurrent Users (Ramp-up)

Overall Status: 🔴 Critical Failure Point Identified

## 1. Executive Summary

The Breakpoint Test confirms that the Teacher Dashboard has a hard capacity limit similar to the Student Dashboard. The system remains stable up to 1,200 concurrent users, after which performance degrades rapidly.

Conclusion: The breaking point is approximately 1,200 - 1,300 VUs. Beyond this threshold, the application server's ability to process requests is overwhelmed by the volume of incoming connections.

### Key Statistics

- Peak VUs: 1,500 (Target)

- Breaking Point: ~1,200 VUs

- Total Requests: ~123,900

- Average Throughput: 413 RPS (Lower than Student Dashboard due to query complexity)

- Response Time (p95): 4.8s (Degraded significantly at peak)

- HTTP Failure Rate: 0.9% (Misleadingly low; masked by long timeouts)

- Functional Check Rate: 62.0% (High failure rate at peak load)

## 2. Aggregate Metrics Summary

The following table summarizes the system's performance over the entire duration of the breakpoint test.

| Metric | Value | Status |
| --- | --- | --- |
| Peak VUs | 1,500 | 🎯 Stress Target |
| Avg Throughput | 413.13 RPS | 🟡 Saturated |
| Response Time (p95) | 4.8s | 🔴 Poor |
| Max Response Time | 45s | 🔴 Critical |
| Blocked Time (Max) | 48s | 🔴 Critical |
| Error Rate (HTTP) | 0.9% | 🟡 Warning |
| Check Pass Rate | 62.0% | 🔴 Major Functional Issues |

## 3. Detailed Findings

### ⚠️ The Breaking Point (~1,200 VUs)

- Throughput Saturation: Throughput flatlined at 413 RPS, unable to scale with the increasing user load. This indicates a resource bottleneck (CPU or DB I/O) preventing faster processing.

- Latency Spike: Latency remained acceptable (<2s) until the 1,200 user mark. As load approached 1,500 users, the 95th percentile response time jumped to 4.8s, and the maximum response time hit 45s.

### ❌ Connection Exhaustion

- Blocked Requests: The http_req_blocked metric peaked at 48s. This confirms that once the system hit its breaking point, new connections were stalled in the backlog queue for nearly a minute.

### ❌ Functional Degradation

- Data Retrieval Failure: A check pass rate of 62.0% confirms that nearly 40% of requests at peak load failed to retrieve or display the correct data, likely due to timeouts or internal application errors not caught by the HTTP status code.

## 4. Root Cause Analysis

1. Database Contention

- The lower throughput (413 RPS) compared to the Student Dashboard suggests that the Teacher Dashboard's database queries are more expensive, causing threads to block longer while waiting for DB results.

2. Connection Queue Overflow

- The high blocked times indicate that the web server's connection limit was reached, causing a backlog of pending requests.

## 5. Recommendations

- Capacity Planning: Rate the Teacher Dashboard for a maximum of 1,000 Concurrent Users.

- Database Indexing: Review and optimize all queries used in the Teacher Dashboard, specifically looking for aggregation queries that can be optimized or indexed.

- Load Shedding: Implement a mechanism to reject new requests (HTTP 503) when the system is at capacity, rather than letting them queue up and time out.